SPECIFICATION

FOR THE

HARRIS RADIO REMOTE CONTROL PROTOCOL (HRRCP)

NUMBER:		10472-7260

REVISION:		1.0

DATE:		September 13, 1995

AUTHOR(S):		Christine Sebastian

SUMMARY:		This document describes the protocol to be used for remote control of the radio communications equipment.

APPROVALS:

Software Engineering Management

Copyright (C) 1981,1983,1993,1994 by Harris Corporation, RF Communications. This specification contains proprietary information of Harris Corporation.  No  disclosure, reproduction, or use of any part thereof may be made without written permission from Harris Corporation.

TABLE OF CONTENTS

1.0  PURPOSE 	  1

2.0  GOALS 	  1

3.0  DIDS 	  1

3.1  FORMAT CONTROL CATEGORY 	  3

3.2  UNIVERSAL CATEGORY	  5

3.3  FREQUENCY CATEGORY	 13

3.4  RECEIVER / TRANSMITTER COMMON CATEGORY	 19

3.5  RECEIVER CATEGORY	 24

3.6  TRANSMITTER CATEGORY	 39

3.7  CHANNEL CATEGORY	 49

3.8  SCAN CATEGORY	 52

3.9  MODEM CATEGORY	 58

3.10  MISCELLANEOUS	 64

4.0  SUMMARY OF DIDS	 65

5.0 GLOSSARY OF TERMINOLOGY	 72

6.0 INDEX OF DIDs	 74

1.0  PURPOSE

The purpose of this document is to describe a protocol to be used for remote control of radio communications equipment. This protocol requires a lower-level protocol, such as HDCP, which will take care of transmitting a packet of data between two pieces of equipment without errors.  This protocol is primarily intended to be used with a multidrop data link, where several slaves are controlled by one master.  The data packet will contain one or more DIDs, as defined below, and each DID may be followed by one data byte.

2.0  GOALS

The primary goals of this protocol are as follows:

1.	To insure maximum compatibility between a variety of controllers and different controlled units.  For example, a newly designed product which conforms to this protocol should be usable with existing controllers without software changes in the controllers.  The controller should not have to know what type of unit it is connected to in order to interpret a readback.

2.	To minimize overhead in critical situations, such as keying, continuous tuning, and scanning.

3.0  DIDS

The following section lists the Data Information Definition (DID) bytes currently defined.  Additional bytes may be defined in the future to meet new requirements.  For compatibility reasons, any DIDs which are not used by the unit that receives them should be ignored.  This way, a unit can report its complete status to a controller which may not necessarily recognize all the DIDs.

Equipment designers and programmers should note that a slave unit in general only needs to recognize DIDs which apply to the particular controller.  For example, if a receiver does not have VFO control there is no need to recognize the VFO DID.  Similarly, if a system design does not require control of a parameter there is no need for the controller to generate or recognize the DID for that parameter.  Exceptions to this rule are certain context switching DIDs which will be explained later.

The protocol is designed in such a way that each DID either stands alone or is followed by a single byte of data.  The least significant bit of the DID is 0 if it stands alone or 1 if it is followed by a data byte.  This scheme enables units which do not recognize a DID to ignore it and still be able to interpret the remainder of the message.  This relies heavily on the lower-level protocol to deliver an error-free message.

Certain DIDs, such as the Program Channel DID, are context switching DIDs.  That is, the occurrence of the DID in a message changes the interpretation of the DIDs which follow. For this reason it is necessary to parse a received HRRCP message in order from the first to the last byte.

The single byte DIDs, especially the key and unkey DIDs, are ideal candidates for the short data messages of HDCP.

In a number of cases DIDs are followed by data bytes where several bits are reserved.  Reserved means that, when the byte is sent, the bit must be 0.  A unit receiving the byte should not expect the bit to be 0, however, and should mask it off.  This will allow compatibility between existing units and future units using HRRCP.

The DIDs are grouped in several categories, by function. Each DID name is followed by a name of 6 characters or less which is used internally by RF Communications to refer to that DID.  The names will be used in this document for examples of DID application.  Parts of data messages will be shown as:

... , didname , didname(data) , didname(data) , ....

where:

'didname' will be the DID name

(data)    will show accompanying data, if any,

in hexadecimal format

Note that all slaves will respond to a DID without data as a request to report data in response to the next poll.

3.1  FORMAT CONTROL CATEGORY

DID:			Shift 1 (SHIFT1)

Application:		All units

Origin:		Masters and slaves

Followed by:		One or more DIDs and data bytes

Meaning:		The Shift 1 DID is used for expansion of the HRRCP protocol.  The DIDs which follow are interpreted according to an alternate set of definitions.  It will be necessary for existing units to recognize and respond to this DID in order to allow compatibility with future products.

Value:			40 hex,   100 octal,   64 decimal

DID:			Shift 2 (SHIFT2)

Application:		All units

Origin:		Masters and slaves

Followed by:		One or more DIDs and data bytes

Meaning:		The Shift 2 DID is used for expansion of the HRRCP protocol.  The DIDs which follow are interpreted according to an alternate set of definitions.  It will be necessary for existing units to recognize and respond to this DID in order to allow compatibility with future products.

Value: 		42 hex,   102 octal,   66 decimal

DID:			Unshift (UNSHFT)

Application:		All units

Origin:		Masters and slaves

Followed by:		Nothing.

Meaning:		The Unshift DID is used after one of the Shift DIDs to return to the basic set of DIDs.  An end of message acts as an implied Unshift DID, so that the next message will be interpreted according to the standard set of DIDs.  It will be necessary for existing units to recognize and respond to this DID in order to allow compatibility with future products.

Value: 		44 hex,   104 octal,   68 decimal

DID:			No-op (NO-OP)

Application:		All units.

Origin:		Masters and slaves.

Followed by:		Nothing or one byte dependent on value used.

Meaning:		All units ignore this DID and its data byte if included.  The DID can be used to hold the place of DIDs which are removed from an assembled message before it is transmitted.

Data Format:		Any data following is ignored, so there are no restrictions.

Value:			00/01 hex,   000/001 octal,   0/1 decimal

3.2  UNIVERSAL CATEGORY

DID:			Self-test (SELTST)

Application:		All units.

Origin:		Master and slaves.

Followed by:		Nothing, or code for test to be performed (from master).  Status byte (from slaves).

Meaning:		The slave should do a self-test of its hardware. When it is through with its self-test, it will respond to the next polling request with the self-test DID and one byte of self-test status.  Note that only one error can be reported at a time, usually the first error detected by the slave.  If the self-test command DID is not followed by a data byte, the slave should do a non-destructive self-test.  If the DID is followed by a code, the slave will suspend normal operation and perform a comprehensive self-test of the indicated function.  During the self-test the slave should respond to polls with the State DID and the Test data code.

Note that some slaves enter a Test state (see STATE DID) upon receipt of the SELTST DID.  If they remain in that state after test is complete and reported, the STATE DID should be sent to command desired operation.

Data Format:		From master:  No codes for self-test command have been defined.  From slaves:

Older products used the following codes:

00 : Self-test passed

01 : PROM fault

02 : RAM fault

03 : Receiver fault

04 : Exciter fault

New products may include the capability of BITE to detect and report faults to module level.  These remotes will report BITE fail (data byte not = 00) OR BITE pass (data = 00) when BITE is remotely commanded, and any failures from local execution of BITE, once, shortly after local BITE has completed.  (see next page)

The data byte sent by the slave with the SELTST DID should be formatted as follows:

/  0	/  1	/  0	/  1	/  1	/  0	/  1	/  0	/

|					|			|

|	5-bit assembly number	|   3-bit fault number

The above data byte, 5A hex, follows the SELTST DID to indicate a BITE failure of Assembly 11, fault 2.  In this fashion, older HRRCP-based units can still use the SELTST data format, essentially reporting faults 1 through 4 in 'assembly 0'.  To handle assemblies with more faults than can be reported in the 3-bit field, the unit could report, as appropriate, 'receiver fault' (03) or 'exciter fault' (04), but to try to carry out BITE's purpose of fault isolation, a better report may consist of the assembly number, and zero as the fault number (i.e., 'Assy n, general fault').  Of course, for all units, a zero assembly number and zero fault number (00) indicates 'all passed'.

Value:			D2/D3 hex, 322/323 octal, 210/211 decimal

DID:			Error (ERROR)

Application:		All units.

Origin:		Slaves.

Followed by:		Error code data byte.

Meaning:		A slave has received a message with some sort of error.  This can be used to report frequency out of bounds, channel number out of bounds, etc.  This allows certain error checking to be done by the slave, where error conditions may vary from unit to unit.

Data format:		The Error DID is followed by an error code which indicates the type of error.  Note that only one error can be reported at a time, usually the first error detected by the slave.  The error code is defined as follows (numbers are in hexadecimal).

01:	Invalid Channel Number

02:	Invalid frequency

03:	Invalid mode

04:	Invalid tuning method/AGC speed

05:	The slave is under local control. No commands can be accepted.

06:	The slave is scanning or frequency hopping. No channels can be programmed.

07:	Remote channel programming was requested but is not supported in the slave device.

08:	Commanded bandwidth was invalid.

09:	Dwell time specified was invalid for the slave.

0A:	Invalid group operation:

Handles any of:

a.	Adding channels to a "full" group.

b.	Deleting a channel not in a group.

c.	Deleting a channel from an "empty" group.

d.	Group operation on a slave which does not support groups.

e.	Invalid group number.

0B:	Invalid data format (for example, invalid BCD following the frequency DIDs, channel DID, etc.)

0C:	State error. Used for invalid state following the State DID, or for invalid operation for the current state. In the latter case this error code indicates that the received command was partially or completely ignored.

0D:	Slave is busy. Part of or all of the command cannot be executed presently. All or part of the command may have been ignored.

Value:			A3 hex,   243 octal,   163 decimal

DID:			Report Complete Status (REPCOM)

Application:		All units.

Origin:		Master

Followed by:		Nothing

Meaning:		Addressed device should ready a data message with its complete status in the form of a series of DIDs and the associated data bytes.  This message should be sent in response to the next polling request for messages.  Note that the slave will NOT send the message in response to the DID, but only when polled later.  Use of the flag bits in the polling message of HDCP is equivalent and preferred, chiefly since the request and data response can be done in one message exchange.

Value:			D0 hex,   320 octal,   208 decimal

DID:			Who Are You (WHORU)

Application:		All slaves.

Origin:		Master or slaves.

Followed by:		Nothing (when from master), or an identification code (when from a slave).

Meaning:		When a slave receives the Who-Are-You DID, it will respond with the Who-Are-You DID and the identification byte on the next polling cycle.  The master will then be able to tell what kind of unit he is talking to, and adjust his commands accordingly.  The use of the flag bits in the polling message of HDCP is equivalent and preferred.

Data format:		From the slave, this DID is followed by an 8-bit number identifying the type of unit.  For equipments which have a range of code numbers, the values may be used to differentiate configurations.  With this information, a controller may be able to know what options are installed and commandable on the slave equipment.  The following types are currently defined (codes are in hexadecimal):

CODE		UNIT

01		RF-550 (with RF-797 LCU)

02		RF-130 (with RF-797 LCU)

03		RF-745 (with RF-797 LCU)

04		RF-270

10-1F		RF-590

20-2F		RF-1310 based transmitters

60		RF-3466

RF-590A sets the MSB if FSK option is installed

Value:			86/87 hex,   206/207 octal,   134/135 decimal

DID:			Fault (FAULT)

Application:		Units with extended fault reporting.

Origin:		Slaves.

Followed by:		One byte of fault data.

Meaning:		The slave uses this DID to report a fault to the master.

Data format:		The Fault DID is followed by a fault data byte of the following format.  A fault DID with a data byte of 0 is used to indicate that a previously reported fault no longer exists, or to report a status of no faults.  Note that only one fault can be reported at a time, usually the first fault detected by the slave.  The fault code is defined as follows (numbers are in hexadecimal).

00:	No fault

01:	Synthesizer out of lock

02:	Controller (LCU) fault (For example, the readback does not agree with the commanded data, or online diagnostics have discovered a fault in the control hardware.)

03:	Fan fault

All other codes are reserved for future products.  Masters should display them as undefined faults, to ensure some compatibility with those products.  For faults that can be isolated to a specific assembly without full BITE execution, the slave may instead send the SELTST DID with the appropriate data, even though a complete BITE execution

may not have occurred.

Value:			A1 hex,   241 octal,   161 decimal

DID:			Spare 1 (SPARE1)

Application:		Units with spare I/O lines.

Origin:		Master or slaves.

Followed by:		One byte of spare command or readback data.

Meaning:		A master uses this DID to send spare data to units which have spare output lines.  A slave which has spare lines uses this DID to report back the state of those lines to the master.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Value:			81 hex,   201 octal,   129 decimal

DID:			Spare 2 (SPARE2)

Application:		Units with spare I/O lines.

Origin:		Master or slaves.

Followed by:		One byte of spare command or readback data.

Meaning:		A master uses this DID to send spare data to units which have spare output lines.  A slave which has spare lines uses this DID to report back the state of those lines to the master.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Value:			83 hex,   203 octal,   131 decimal

DID:			Alternate Operation (ALTERN)

Application:		All units

Origin:		Master and slaves.

Followed by:		Nothing

Meaning:		This DID is used with equipment having more than one major type of operation.  The alternate operation may be a hopping, secure, antijam, or other distinctly different type than the normal use of the equipment.  This is specific to the equipment type in question.  A master uses this DID to command a slave to enter alternate operation.  The slave uses it to report alternate operation to a master.  While in the alternate operation, command/status handling may be different than in normal operation.

Value:			D8 hex,   330 octal,   216 decimal

DID:			Normal Operation (OPNORM)

Application:		All units

Origin:		Master and slaves.

Followed by:		Nothing

Meaning:		This DID is used with equipment having more than one major type of operation.  The alternate operation may be a hopping, secure, antijam, or other distinctly different type than the normal use of the equipment.  This is specific to the equipment type in question.  A master uses this DID to command a slave to exit alternate operation and return to normal operation.  The slave uses it to report normal operation to a master.  While in the alternate operation, command/status handling may be different than in normal operation.  A master may want to request complete status from a slave that has just exited alternate operation.

Value:			DA hex,   332 octal,   218 decimal

3.3  FREQUENCY CATEGORY

DID:			10 MHz/1 MHz Frequency (FREQ3)

Application:		All units.

Origin:		Master and slaves.

Followed by:		One byte of 10 MHz/1 MHz frequency data.

Meaning:		A master uses this DID to send new frequency data to a slave.  The slave uses the DID to report frequency status, whether remotely or locally commanded.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Data format:		The DID is followed by a byte which consists of two packed BCD digits.

There may be restrictions on the maximum or minimum data value, as many units do not span the 00 to 99 MHz range.

Value:			C1 hex,   301 octal,   193 decimal

DID:			100 KHz/10 KHz Frequency (FREQ2)

Application:		All units.

Origin:		Master and slaves.

Followed by:		One byte of 100 KHz/10 KHz frequency data.

Meaning:		A master uses this DID to send new frequency data to a slave.  The slave uses the DID to report frequency status, whether remotely or locally commanded.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Data format:		The DID is followed by a byte which consists of two packed BCD digits.

Value:			C3 hex,   303 octal,   195 decimal

DID:			1 KHz/100 Hz Frequency (FREQ1)

Application:		All units.

Origin:		Master and slaves.

Followed by:		One byte of 1 KHz/100 Hz frequency data.

Meaning:		A master uses this DID to send new frequency data to a slave.  The slave uses the DID to report frequency status, whether remotely or locally commanded.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Data format:		The DID is followed by a byte which consists of two packed BCD digits.

Value:			C5 hex,   305 octal,   197 decimal

DID:			10 Hz/1 Hz Frequency (FREQ0)

Application:		All units.

Origin:		Master and slaves.

Followed by:		One byte of 10 Hz/1 Hz frequency data.

Meaning:		A master uses this DID to send new frequency data to a slave.  The slave uses the DID to report frequency status, whether remotely or locally commanded.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Data format:		The DID is followed by a byte which consists of two packed BCD digits.

Value:			C7 hex,   307 octal,   199 decimal

Note that the frequency DIDs are independent.  That is, it is possible for a master to send a slave just one of the DIDs, to change just one or two digits of the frequency.

Example: A frequency of 12.345678 MHZ would be sent as

..., FREQ3(12), FREQ2(34), FREQ1(56), FREQ0(78), ...

and a change of this value to 12.378678 MHZ would be indicated by sending

..., FREQ1(86), FREQ2(37), ...

DID:			VFO High (VFOHI)

Application:		Receivers, receiver controllers, and transceivers with remote VFO capabilities.

Origin:		Masters and slaves.

Followed by:		A byte with sign and the 1 KHz VFO digit.

Meaning:		A master uses this DID to send new VFO data to a slave.  The slave uses the DID to report its VFO frequency.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Data format:		The upper four bits are 0000 for positive frequency, 1111 for negative.  The lower four bits hold the BCD value for the 1 KHz digit.

Value:			BD hex,   275 octal,   189 decimal

DID:			VFO Low (VFOLOW)

Application:		Receivers and receiver controllers with remote VFO capabilities.

Origin:		Master and slaves.

Followed by:		Data byte with 100 Hz and 10 Hz VFO frequency data.

Meaning:		A master uses this DID to send new VFO data to a slave.  A slave uses it to report its current VFO frequency.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Data format:		The data byte consists of two packed BCD digits.

Value:			BF hex,   277 octal,   191 decimal

Note that the VFO DIDs are independent.  That is, the VFO Low DID could be sent without sending the VFO High DID, if only the least significant digits were to be changed.

Example: A VFO of 6.40 KHZ would be sent as

..., VFOHI(06), VFOLOW(40), ...

and -0.25 is sent as

..., VFOHI(F0), VFOLOW(25), ...

DID:			BFO High (BFOHI)

Application:		Receivers, receiver controllers, and transceivers with remote BFO capabilities.

Origin:		Masters and slaves.

Followed by:		Data byte with sign and the 1 KHz BFO digit.

Meaning:		A master uses this DID to send new BFO data to a slave.  The slave uses the DID to report its BFO frequency.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Data format:		The upper four bits are 0000 for positive frequency, 1111 for negative.  The lower four bits hold the BCD value for the 1 KHz digit.

Value:			B3 hex,   263 octal,   179 decimal

DID:			BFO Low (BFOLOW)

Application:		Receivers and receiver controllers with remote BFO capabilities.

Origin:		Master and slaves.

Followed by:		Data byte with 100 Hz and 10 Hz BFO frequency data.

Meaning:		A master uses this DID to send new BFO data to a slave.  A slave uses it to report its current BFO frequency.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Data format:		The data byte consists of two packed BCD digits.

Value:			B5 hex,   265 octal,   181 decimal

Note that the BFO DIDs are independent.  That is, the BFO Low DID could be sent without sending the BFO High DID, if only the least significant digits were to be changed.

Example: A BFO of 1.30 KHZ would be sent as

..., BFOHI(01), BFOLOW(30), ...

and -7.51 is sent as

..., BFOHI(F7), BFOLOW(51), ...

3.4  RECEIVER / TRANSMITTER COMMON CATEGORY

DID:			State (STATE)

Application:		Units with several possible operating states.

Origin:		Masters and slaves.

Followed by:		One byte of state data.

Meaning:		A master uses this DID to command state of a slave.  The slave uses the DID to report its current state to the master.

Data Format:		The data byte is an 8-bit code which indicates the state of the radio.  The following states are currently defined:

01: Normal operation (receive or transmit on fixed frequency)

02: Channel Scan

03: Group Scan

04: Test (used for status only)

Note that if a slave is commanded to channel scan or group scan state with the State DID, it will not actually start scanning until the Resume Scan DID is received.  The Resume Scan DID could be contained in the same message (following the State DID), or it could be sent later.

Value:			CF hex,   317 octal,   207 decimal

DID:			Mode (MODE)

Application:		All units.

Origin:		Master and slaves.

Followed by:		One byte of mode data.

Meaning:		A master uses this DID to send new emission/demodulation mode data to a slave.  The slave uses it to report its current mode.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Data Format:		The DID is followed by a data byte with the following format:

Bits 0-3: MODE CODE

Bits 4-7: Reserved

MODE		CODE

CW			0

AFSK (RATT)	1

AME/AM		2

USB			3

LSB			4

2ISB			5

4ISB			6

MCW			7

FM			8

FSK			9

Value:			C9 hex,   311 octal,   201 decimal

Example: AM operation is indicated by sending

..., MODE(02), ...

DID:			Command/Status (COMSTA)

Application:		All units.

Origin:		Master and slaves.

Followed by:		One byte of command information (when sent by master).  One byte of status information (when sent by slave).

Meaning:		A master uses this DID to send command data (standby, operate, etc.) to a slave.  A slave uses it to send status information to the master.

Data Format:		COMMAND WORD (TRANSMITTERS AND TRANSCEIVERS ONLY):

Bit 0: TUNE=HI

Bit 1: OPERATE=HI

Bit 2: STANDBY=HI

Bit 3: AMP OFF=HI

Bit 4 - 7: Reserved

STATUS WORD:

Bit 0: TUNING=HI

Bit 1: OPERATE=HI

Bit 2: STANDBY=HI

Bit 3: AMP OFF=HI

Bit 4: WARMING UP=HI

Bit 5: READY=HI

Bit 6: CHANNELIZED=HI

Bit 7: LOCAL=HI

NOTES:

Bits 0 through 5 of the status word are used by transmitters and transceivers only.  Bit 5 (Ready) and Bit 0 (Tune) both low indicates that the transmitter requires tuning.  The bits in the status word are non-exclusive;  except for bits 1 through 3, more than one bit can be set at one time.

Value:			CB hex,   313 octal,   203 decimal

Example: The remote transmitter reporting Operate, untuned, sends

..., COMSTA(02), ...

To tune it, the master commands

..., COMSTA(03), .. to which slave responds

..., COMSTA(03), ..; and later ..., COMSTA(22), ...

DID:			USB Line Audio Level (USBLIN)

Application:		RF-590A family of receivers

Origin:		Masters and slaves

Followed by:		Nothing when sent by master, or binary line audio voltage when sent by a slave.

Meaning:		The slave will always send this DID and data in response to a poll for complete status.  The master may send this DID without data to force the slave to report in response to a poll for non-urgent status.

Data Format:		The data is an unsigned byte as read from the receivers A/D converter.

Channel contexts:

Value:			30/31 hex,   60/61 octal,   48/49 decimal

DID:			LSB Line Audio Level (LSBLIN)

Application:		RF-590A family of receivers

Origin:		Masters and slaves

Followed by:		Nothing when sent by master, or binary line audio voltage when sent by a slave.

Meaning:		The slave will always send this DID and data in response to a poll for complete status.  The master may send this DID without data to force the slave to report in response to a poll for non-urgent status.  NOTE:  The data reported by the slave is not valid if the A18 (ISB) assy is not installed.

Data Format:		The data is an unsigned byte as read from the receivers A/D converter.

Channel contexts:

Value:			32/33 hex,   62/63 octal,   50/51 decimal

3.5  RECEIVER CATEGORY

DID:			Receiver Command/Status (RXCMST)

Application:		Receivers, receiver controllers, and transceivers.

Origin:		Masters and slaves

Followed by:		One byte of command or status data.

Meaning:		A master uses this DID to control several receiver parameters.  A slave uses the DID to report the status of those parameters.

Data format:		The DID from master to slave is followed by a command byte as follows:

Bit 0: HI=Receiver mute

Bit 1: HI=Break lock (AFC)

Bit 2: HI=AFC memory disable

Bit 3: HI=Noise blanking

Bit 4: HI=Preselector bypass (Internal to Receiver)

Bits 5-7: Reserved

From slaves to masters, the status byte is as follows:

Bit 0: HI=Receiver mute

Bit 1: HI=AFC lock indicator

Bit 2: HI=AFC memory indicator

Bit 3: HI=Noise blanking

Bit 4: HI=Preselector bypass (Internal to Receiver)

Bits 5-7: Reserved

Value:			A9 hex,   251 octal,   169 decimal

Example:

A muted receiver reports

..., RXCMST(01), ...

To activate the noise blanker, the master sends

..., RXCMST(09), ...

DID:			RF Gain (RFGAIN)

Application:		Receivers and receiver controllers with remote RF gain capabilities.

Origin:		Master and slaves.

Followed by:		One byte of RF gain data.

Meaning:		A master uses this DID to send new RF gain data to a slave.  The slave uses it to report its current RF gain value.

Data format:		The data is encoded as 0 for minimum gain and FF hex for maximum gain.  All 256 data values are valid.  Note that a certain slave may use this data in a linear or logarithmic fashion.

Value:			BB hex,   273 octal,   187 decimal

DID:			Bandwidth (BANDWT)

Application:		Receivers, receiver controllers, and transceivers with remote bandwidth selection capabilities.

Origin:		Masters and slaves.

Followed by:		One byte of bandwidth data.

Meaning:		A master uses this DID to change the IF (filter) bandwidth of the slave.  A slave uses it to report its current IF bandwidth.  Note that, for some receivers a mode change may also imply a bandwidth change.  In this case a message from the master to the slave including a mode command may also require a bandwidth command, to ensure that the resulting bandwidth is the one desired.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Data format:		The bandwidth is identified by a one-byte value.  The current definitions are listed below.

Data	Bandwidth_(KHz)			Associated_Modes

00	Blank (No filter)

01	0.15					CW/FSK

02	0.15					FSK

03	0.17					CW

04	0.25					CW

05	0.3					CW/FSK

06	0.4					CW

07	0.5					CW

08	0.75					CW

09	0.85					FSK

0A	1.0					CW/FSK

0B	2.0					FM/AM

0C	2.7					LSB/USB/FSK

0D	3.2					AM

0E	3.2					LSB/USB/FSK

0F	4.0					AM

10	6.0					AM

11	6.0					LSB/USB/FSK

12	6.8					AM

13	8.0					AM

14	16.0					AM/FM

15	20.0					AM/FM

16	Special A				AM

17	Special A				CW

18	Special B				AM

19	Special B				CW

1A	Special C				AM

1B	Special C				CW

1C	2.8					LSB/USB/FSK

1D	10.0					AM

1E	2.7					LSB/USB/CW/FSK

1F	10.0					AM/CW/FSK

20	6.0					FM/CW/FSK

21	5.0					LSB/USB

22	3.2					AM/FSK

23	2.7					LSB/USB

24	2.7					LSB/USB/AM/CW

Value:			B1 hex,   261 octal,   177 decimal

BANDWT example: A receiver is commanded to 6.0 KHz bandwidth in AM mode by sending

..., MODE(02), BANDWT(10), ...

The receiver in CW mode may send

..., MODE(00), BANDWT(05), ...

DID:			Bandwidth Configuration (BWCONF)

Application:		Receivers, receiver controllers, and transceivers with several possible bandwidth configurations (eg., RF-590).

Origin:		Masters and Slaves.

Followed by:		One byte of bandwidth configuration data.

Meaning:		A master uses this DID to request a report of the bandwidth configuration of a slave (no data).  A slave uses this DID to report its bandwidth configuration (with data).

Data format:		The Bandwidth Configuration DID uses the same data codes as the Bandwidth DID.  A slave sends a Bandwidth Configuration DID and a data byte for each IF bandwidth filter which is installed.  I.E., if a receiver has eight filters installed, it will send eight Bandwidth Configuration DIDs, each followed by data specifying that filter.  A controller can use this information to allow only those bandwidths available to be commanded.

Value:			88/89 hex,   210/211 octal,   136/137 decimal

Example: The master requiring configuration information sends

..., BWCONF , ...

To which a future receiver response will contain

..., BWCONF(1C),..., BWCONF(05), BWCONF(12), ...

DID:			Tuning Method/AGC Speed (TUNAGC)

Application:		Receivers and receiver controllers.

Origin:		Master and slaves.

Followed by:		One byte of tuning method (VFO, BFO,VFO/BFO, Fixed) and AGC speed data.

Meaning:		The master uses this DID to send AGC speed/tuning method data to a slave.  The slave uses it to report its current Tuning Method/AGC speed status.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Data Format:

Bit 0: TUNING A

Bit 1: TUNING B

Bit 2: TUNING C

Bit 3: Reserved

Bit 4: AGC A

Bit 5: AGC B

Bit 6: AGC C

Bit 7: Reserved

AGC SPEED 	C  B  A		TUNING_METHOD	C  B  A

SLOW       	0  0  0          	FIXED          	0  0  0

FAST       	0  0  1          	NOT USED       	0  0  1

MEDIUM    	0  1  0	         	NOT USED       	0  1  0

EXTERNAL	0  1  1          	NOT USED       	0  1  1

COHERENT	1  0  0          	BFO       	     	1  0  0

DATA       	1  0  1          	VFO/BFO        	1  0  1

NOT USED  	1  1  0          	VFO            		1  1  0

OFF        	1  1  1          	AFC            		1  1  1

Value:			B7 hex,   267 octal,   183 decimal

Example: A receiver setup to medium AGC, BFO control, will readback

..., TUNAGC(24), ...

Note:	A receiver not supporting separate BFO enable (e.g. RF-590 series) will use a tuning method - fixed (000)

DID:			Clarifier (CLARFY)

Application:		Units with internal clarifier counters.

Origin:		Master and slaves.

Followed by:		One byte of clarifier data.

Meaning:		From the master:  This DID is a clarifier command DID.  It starts the clarifier incrementing or decrementing.  The rate is controlled in the radio, either in hardware or software.  It also stops the clarifier from incrementing or decrementing.  From the slave: This DID reports the clarifier status.  The data byte tells whether the clarifier is above or below zero.

Data Format:		Bits 0 through 2 are used from the master to the slave as command bits.  Bits 3 and 4 are used from the slave to the master as status bits.  The clarifier is incremented or decremented by a counter in the radio. When the slave gets the clarifier DID with the increment clarifier bit (Bit 0) set, it will increment its clarifier until it gets the clarifier DID with the stop clarifier bit (Bit 2) set.

BIT 0: INCREMENT CLARIFIER = HI (Command)

BIT 1: DECREMENT CLARIFIER = HI (Command)

BIT 2: STOP CLARIFIER = HI (Command)

BIT 3: HI CLARIFIER = HI (Readback)

BIT 4: LO CLARIFIER = HI (Readback)

BIT 5: Reserved

BIT 6: Reserved

BIT 7: Reserved

Value:			B9 hex,   271 octal,   185 decimal

Example: The master starts increasing frequency with a clarifier command

..., CLARFY(01), ...

a later report from slave includes

..., CLARFY(08), ...

and the master may stop the process by commanding

..., CLARFY(04), ...

DID:			USB AGC Level (USBAGC)

Note: This DID is also known as USBOUT.

Application:		RF-590A family of receivers

Origin:		Masters and slaves

Followed by:		Nothing when sent by master, or binary AGC voltage when sent by a slave.

Meaning:		The slave will always send this DID and data in response to a poll for complete status.  The master may send this DID without data to force the slave to report in response to a poll for non-urgent status.

Data Format:		The data is an unsigned byte as read from the receivers A/D converter.

Channel contexts:

Value:			50/51 hex,   120/121 octal,   80/81 decimal

DID:			LSB AGC Level (LSBAGC)

Note: This DID is also known as LSBOUT.

Application:		RF-590A family of receivers

Origin:		Masters and slaves

Followed by:		Nothing when sent by master, or binary AGC voltage when sent by a slave.

Meaning:		The slave will always send this DID and data in response to a poll for complete status.  The master may send this DID without data to force the slave to report in response to a poll for non-urgent status.  NOTE: The data reported by the slave is not valid if the A18 (ISB) assy is not installed.

Data Format:		The data is an unsigned byte as read from the receivers A/D converter.

Channel contexts:

Value:			52/53 hex,   122/123 octal,   82/83 decimal

DID:			FSK Shift & Shift Polarity (FSHIFT)

Application:		RF-590A family of receivers with FSK option.

Origin:		Masters and slaves

Followed by:		Data byte containing shift value and shift polarity.

Meaning:		Used by a master to command a particular FSK shift and shift polarity, and used by a slave to report the current FSK shift and polarity.

Data Format:		The data byte is bit mapped with the shift value contained in the 2 lower bits and the polarity contained in the high bit.  See below.

ÚÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄ¿

³     ³     ³     ³     ³     ³     ³     ³     ³

³bit 7³     ³     ³     ³     ³     ³     ³bit 0³

ÀÄÄÂÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÙ

³

³    \                        /    \       /

³                 ³                    ³

³                 ³                    ÀÄÄÄÄÄÄÄÄ FSK SHIFT

³                 ³

³                 ³                              00 = Very Narrow (VN)

³                 ³                              01 = Narrow (N)

³                 ³                              10 = Medium (M)

³                 ³                              11 = Wide (W)

³                 ³

³                 ÀÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ NOT USED (RESERVED

³

³                                                Will always be reported

³                                               by slave as zeroes.

³

³

ÀÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ POLARITY BIT

0 = Normal   (+)

1 = Inverted (-)

Value:			38/39 hex,   70/71 octal,   56/57 decimal

DID:			FSK Center Freq Low (FCFLOW)

Application:		RF-590A family of receivers with FSK option.

Origin:		Masters and slaves

Followed by:		Data byte containing 10 Hz and 1 Hz FSK center freq digits.

Meaning:		Used by a master to command a particular FSK center frequency and used by a slave to report the current FSK center frequency.  The slave always replies with this DID as a response to a poll for complete, and will also reply with this DID as a response to a non-urgent poll if the status has changed.

Data Format:		The data byte contains two packed BCD digits.  Bits 4-7 contains the 10 Hz digit, and bits 0-3 contains the 1 Hz digit.

Value:			34/35 hex,   64/65 octal,   52/53 decimal

DID:			FSK Center Freq High (FCFHI)

Application:		RF-590A family of receivers with FSK option.

Origin:		Masters and slaves

Followed by:		Data byte containing 1 kHz and 100 Hz center frequency digits as well as the center frequency sign.

Meaning:		Used by a master to command a particular FSK center frequency and used by a slave to report the current FSK center frequency.  The slave always replies with this DID as a response to a poll for complete, and will also reply with this DID as a response to a non-urgent poll if the status has changed.

Data Format:		The data byte contains two packed BCD digits as well as a sign bit.  The 1 kHz digit ranges from 0 to 3 (2 bits).  See below.

ÚÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄ¿

³     ³     ³     ³     ³     ³     ³     ³     ³

³bit 7³     ³     ³     ³     ³     ³     ³bit 0³

ÀÄÄÂÄÄÁÄÄÂÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÙ

³     ³

³     ³     \      /    \                  /

³     ³        ³                 ³

³     ³        ³                 ÀÄÄÄÄÄÄÄÄÄÄÄ 100 Hz Digit (BCD)

³     ³        ³

³     ³        ³

³     ³        ÀÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ 1 kHz Digit (0-3)

³     ³

³     ³

³     ³

³     ÀÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ Not used (Reserved)

³

³

³

ÀÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ SIGN BIT

0 = Positive   (+)

1 = Inverted (-)

Value:			36/37 hex,   66/67 octal,   54/55 decimal

DID:			Options (OPTION)

Application:		RF-590A family of receivers with FSK option, or control and status of preselector antenna option.

Origin:		Masters and slaves

Followed by:		Data byte containing OPTION NUMBER and OPTION VALUE from master or slave.  A master may also send the OPTION DID only, to a slave to cause the slave to report all options as a response to a non-urgent poll.

Meaning:		Used by a master to command a particular option or to request the slave to report all available options.  Used by a slave to report option settings.  When an options value change, the new value will be reported to the master as a response to a non-urgent poll.  If the OPTION DID is sent to the slave without data, the slave will respond to the next non-urgent poll with a list of all options values.  The master should relate the options to the equipment ID as only 32 options are available.  In the future, OPTION # 0 on a RF-1310 may mean something entirely different for OPTION # 0 on a RF-590.

Data Format:		The data byte contains the option number as the upper 5 bits in the data byte, and the selection as the low 3 bits.  This allows up to 32 different options with up to 8 selections for each option.

ÚÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄ¿

³     ³     ³     ³     ³     ³     ³     ³     ³

³bit 7³     ³     ³     ³     ³     ³     ³bit 0³

ÀÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÙ

\                      /    \             /

³                    ³

³                    ÀÄÄÄÄÄÄÄÄ Option's Selection

³                              Range: 0 - 7

³

³

ÀÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ Option's Number

Range: 0 - 31

Value:			3A/3B hex,   72/73 octal,   58/59 decimal

CURRENT RF-590A OPTIONS:

OPTION NUMBER            		OPTION SELECTIONS

#0   FSK VN SHIFT BAUD		#0	 50 BAUD FILTER   <--- DEFAULT

#1	100 BAUD FILTER

#2	200 BAUD FILTER

#1   FSK N SHIFT BAUD		#0	 50 BAUD FILTER   <--- DEFAULT

#1	100 BAUD FILTER

#2	200 BAUD FILTER

#2   FSK M SHIFT BAUD		#0	 50 BAUD FILTER   <--- DEFAULT

#1	100 BAUD FILTER

#2	200 BAUD FILTER

#3   FSK W SHIFT BAUD		#0	 50 BAUD FILTER   <--- DEFAULT

#1	100 BAUD FILTER

#2	200 BAUD FILTER

#4   FSK AGC			#0	SLOW

#1	FAST              <--- DEFAULT

#2	MEDIUM

#3	OFF

#4	DATA

#5   ANTENNA POSITION		#0	MAIN ANTENNA

#1	AUX ANTENNA

NOTE:	The "DEFAULT" selections shown are the values that are initialized when a channel memory (battery backed up RAM) failure occurs.

DID:			Preselector Status (PRESTA)

Application:		All units

Origin:		Masters and slaves

Followed by:		Nothing, or one byte of command or status data.

Meaning:		This DID is used to control one or more miscellaneous functions of a slave.  These may include on/off selection of several different device features.  A master uses this DID to request status, or to send data to a slave.  The slave uses it to report its current setup.  This DID should be sent in response to a poll for complete status as well as when requested.

Data Format:		The DID is followed by a data byte coded specifically for the equipment type involved.  It may be partially or completely bit-mapped.  Check applications for specific equipments.

The data may be different for each slave type, the HFRG R2368/RF-591A pair, the data format is as follows:

Command Format:	bit 0 - commanding antenna position

HI - normal antenna position

LO - auxiliary antenna position

bit 1 - implement command in bit 0 or ignore

HI - implement the command

LO - ignore the command

bit 2 - commanding of preselector bypass

HI - preselector in RF path

LO - preselector in bypass

bit 3 - implement the command

HI - implement the command

LO - ignore the command

bit 4 - presence of preselector (status only)

HI - preselector present

LO - preselector not present

bit 5-7 - unused (set to 0)

Status Format:		bit 0 - antenna position status

HI - normal antenna position

LO - auxiliary antenna position

bit 1 - implement command in bit 0 or ignore

always zero

bit 2 - preselector bypass status

HI - preselector in RF path

LO - preselector in bypass

bit 3 - implement command in bit 2 or ignore

always zero

bit 4 - presence of preselector (status only)

HI - preselector present

LO - preselector not present

bit 5-7 - unused (set to 0)

Value:			5C/5D hex,   134/135 octal,   92/93 decimal

DID:			Signal Detect (SIGDET)

Application:		Receivers

Origin:		Masters and slaves

Followed by:		Nothing, or one byte of command or status data.

Meaning:		A master used this DID to command the receiver to go into Signal Detect mode and to return results.  The slave used it to report its current signal detect status.  The Receiver resumes normal operation after it collects signal detect information.

Data Format:		The data format for the DID from master to slave is unused and must be 0.  The data format from slaves to masters is as follows:

Bit 0:		HI=Test  Frequency #1 has detected signal.

Bit 1:		HI=Test  Frequency #2 has detected signal.

Bit 2:		HI=Test  Frequency #3 has detected signal.

Bit 3:		HI=Test  Frequency #4 has detected signal.

Bit 4:		HI=Test  Frequency #5 has detected signal.

Bit 5-7:	Reserved (set to 0)

Value:			54/55 hex,   124/125 octal,   84/85 decimal

3.6  TRANSMITTER CATEGORY

DID:			Key (KEY)

Application:		Transmitters and transceivers.

Origin:		Master and slaves.

Followed by:		Nothing

Meaning:		When sent from a master to a transmitter or transceiver, it will key the transmitter.  When sent from a transceiver or transmitter to a master, it means that the transmitter is keyed.

Value:			D4 hex,   324 octal,   212 decimal

DID:			Unkey (UNKEY)

Application:		Transmitters and transceivers.

Origin:		Masters and slaves.

Followed by:		Nothing

Meaning:		A transmitter controller which receives this DID will unkey its transmitter.  A transmitter controller which had been keyed will send it when the keyline goes inactive.

Value:			D6 hex,   326 octal,   214 decimal

DID:			Carrier (CARRIR)

Application:		Transmitters and transceivers with carrier suppression control.

Origin:		Masters and slaves.

Followed by:		One byte of carrier data as defined below.

Meaning:		A master uses this DID to send new carrier suppression data to a slave.  A slave uses it to report the current carrier status.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Data format:	The carrier DID is followed by a data byte with the following format:

Bits 0-2: Carrier selection value

Bits 3-7: Reserved

CARRIER		VALUE

-10dB			0

-06dB			1

-20dB			2

-INF 			3

-16dB			4

-26dB			6

-06dB (status only)	5

-INF (status only)	7

Value:			CD hex,   315 octal,   205 decimal

DID:			Power-up (PWRUP)

Application:		Exciters and exciter controllers.

Origin:		Slaves.

Followed by:		Transmitter status prior to when the power was turned off.

Meaning:		This is the first polling request since the slave powered up.

Data format:		The data format for the power-up DID is the same as that for the Command/Status (COMSTA) DID.

Value:			85 hex,   205 octal,   133 decimal

DID:			Transmitter Command/Status (TXCMST)

Application:		Exciters and exciter controllers.

Origin:		Masters and slaves

Followed by:		One byte of command or status data.

Meaning:		A master uses this DID to control several transmitter/exciter parameters.  A slave uses the DID to report the status of those parameters.

Data format:	The DID is followed by a byte as follows:

Bit 0: HI=Clipper on

Bit 1: HI=Vox on

Bit 2: HI=AntiVox on

Bits 3-7: Reserved

From master to slave, this is a command.  From slaves to masters, this is a status report.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Value:			AB hex,   253 octal,   171 decimal

DID:			Audio Select (AUDIO)

Note: This DID has been renamed 'USBAUD'

Value:			AF hex,  257 octal,   175 decimal

DID:			USB Audio Select (USBAUD)

Application:		All units

Origin:		Masters and slaves

Followed by:		One data byte, command or status data

Meaning:		A master uses this DID to send new USB audio selection commands to a slave.  The slave uses it to report USB audio selection.  The DID is used to specify a desired use of audio inputs or outputs on a controlled unit.  For exciters, the audio select determines which of several audio inputs will be used to go out through the USB transmit path.  For a receiver, the audio output are controlled.

Data format:		The DID is always followed by a data value.  This value is dependant on the unit in question.

For the HFRG Exciter, the data format is as follows:

0 = MUTE

1 = MIC

2 = AUX1

3 = AUX2

4 = AUX3

5 = USB

6 = REMOTE AUDIO

7 = LSB

8 = TTN 	(not used in HFRG Exciter. Used in commercial version)

9 = TTR	(not used in HFRG Exciter. Used in commercial version)

10 = AFN	(AFSK narrow frequency shift)

11 = AFW	(AFSK wide frequency shift)

Notes:

-	AFN, AFW, TTN and TTR audio inputs are only valid when the AFSK board is installed in the exciter.

-	When the AFSK keyer board is installed, the AUX3 audio input is not available in any mode.

-	AFN, AFW, TTN and TTR audio inputs are only available in USB, LSB, ISB, and AFSK modes of operation.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Value:			AF hex,  257 octal,   175 decimal

DID:			LSB Audio Select (LSBAUD)

Application:		All units

Origin:		Masters and slaves

Followed by:		One data byte, command or status data

Meaning:		A master uses this DID to send new LSB audio selection commands to a slave.  The slave uses it to report LSB audio selection.  The DID is used to specify a desired use of audio inputs or outputs on a controlled unit.  For exciters, the audio select determines which of several audio inputs will be used to go out through the LSB transmit path.  For a receiver, the audio output are controlled.

Data format:		The DID is always followed by a data value.  This value is dependant on the unit in question.  For the HFRG Exciter, the data format is as follows:

0 = MUTE

1 = MIC

2 = AUX1

3 = AUX2

4 = AUX3

5 = USB

6 = REMOTE AUDIO

7 = LSB

8 = TTN 	(not used in HFRG Exciter. Used in commercial version)

9 = TTR	(not used in HFRG Exciter. Used in commercial version)

10 = AFN	(AFSK narrow frequency shift)

11 = AFW	(AFSK wide frequency shift)

Notes:

-	AFN, AFW, TTN and TTR audio inputs are only valid when the AFSK board is installed in the exciter.

-	When the AFSK keyer board is installed, the AUX3 audio input is not available in any mode.

-	AFN, AFW, TTN and TTR audio inputs are only available in USB, LSB, ISB, and AFSK modes of operation.

Channel contexts:	This DID can also be used to program or report channel data when following a Program Channel (PROCHA) or Report Channel (REPCHA) DID, respectively.

Value:			A7 hex,  247 octal,   167 decimal

DID:			Transmitter Bypass (TXBYPS)

Application:		Transceivers, transmitters and transmitter controllers

Origin:		Masters and slaves

Followed by:		One data byte, command or status data

Meaning:		A master uses this DID to send bypass commands to a slave.  The slave uses it to report bypass status.  The DID controls the bypass of power amplifiers or couplers in transmitting systems.  The bypass may be done for lower power output, or to switch out an element (i.e.  antenna coupler) for some configuration.

Data Format:		This value will be one of two states.  Data of 00 is used to command and status the 'no bypass' condition.  Data = FFhex commands or shows status of bypass.

Channel contexts:	This DID can also be used to program or report channel data when following a ProgramChannel (PROCHA) or Report Channel (REPCHA) DID, respectively.  Check applications information for specific products.

Value:			8D hex,   215 octal,   141 decimal

DID:			Tune Mode (TUNMOD)

Application:		Exciters

Origin:		Masters and slaves

Followed by:		Nothing (from Master), or one byte of command or status data (from master or slave)

Meaning:		This DID is used to control the tune mode of the slave.  A master uses this DID to request status, or to send command data to a slave.  The slave used it to report its current setup.  This DID should be sent in response to a poll for complete status as well as when requested.  If the Tune Mode changes, this DID should be sent in response to a poll for non-urgent data.

Data Format:		00H indicates normal tune mode,

01H indicates silent tune mode,

Any other patterns are reserved.

Value:			FA/FB hex,   372/373 octal,   250/251 decimal

DID:			Transmit Command (TXCOMD)

Application:		Exciters

Origin:		Master

Followed by:		1 byte of data

Meaning:		Transmit command to set CLIP and VOX on or off.

Data Format:		1st bit 0000 0001B - Clip on

0000 0000B - Clip on

2nd bit 0000 0010B - VOX on

0000 0000B - VOX off

Value:			AA hex,   252 octal,   170 decimal

DID:			Power Output Level (POWER)

Application:		Exciters

Origin:		Master

Followed by:		1 byte of data

Meaning:		This changes the power output level of the exciter.  The data byte sets the manual power attenuation level.

Data Format:		1 byte - 0 - 50 range.  The byte is value representing the desired output power attenuation level.  The range is 0 to 50 representing 0 dB to -50 dB.

Value:			AC/AD hex,   254/255 octal,   172/173 decimal

DID:			Options (OPTION)

Application:		RF-1310 family of Exciters. (See RECEIVER CATAEGORY for RF-590A options.)

Origin:		Masters and slaves

Followed by:		Data byte containing OPTION NUMBER and OPTION VALUE from master or slave.  A master may also send the OPTION DID only, to a slave to cause the slave to report all options as a response to a non-urgent poll.

Meaning:		Used by a master to command a particular option or to request the slave to report all available options.  Used by a slave to report option settings.  When an options value change, the new value will be reported to the master as a response to a non-urgent poll.  If the OPTION DID is sent to the slave without data, the slave will respond to the next non-urgent poll with a list of all options values.  The master should relate the options to the equipment ID as only 32 options are available.  In the future, OPTION # 0 on a RF-1310 may mean something entirely different for OPTION # 0 on a RF-590.

Data Format:		The data byte contains the option number as the upper 5 bits in the data byte, and the selection as the low 3 bits.  This allows up to 32 different options with up to 8 selections for each option.

ÚÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄÂÄÄÄÄÄ¿

³     ³     ³     ³     ³     ³     ³     ³     ³

³bit 7³     ³     ³     ³     ³     ³     ³bit 0³

ÀÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÁÄÄÄÄÄÙ

\                      /    \             /

³                    ³

³                    ÀÄÄÄÄÄÄÄÄ Option's Selection

³                              Range: 0 - 7

³

³

ÀÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ Option's Number

Range: 0 - 31

Value:			3A/3B hex,   72/73 octal,   58/59 decimal

CURRENT RF-1310A OPTIONS:

OPTION NUMBER            		OPTION SELECTIONS

#1   FSK SHIFT 			#0	 85 Hz   <--- DEFAULT

#1	170 Hz

#2	425 Hz

#3	850 Hz

Note:

The options definitions have different meanings in the Exciter and Receiver.

This option is NOT supported in HFRG (RF1310A-37) Exciters.

DID:			PA Assembly (PAASSY)

Application:		Exciter

Origin:		Slave

Followed by:		1 byte of data

Meaning:		At the end of BITE or when a run-time fault is detected by the PA it returns an assembly number and a fault number to the Exciter.  The Exciter breaks this up and sends it as two messages.

Data Format:		Assembly number (1 byte)

Value:			58/59 hex,   130/131 octal,   88/89 decimal

DID:			PA Fault (PAFLT)

Application:		Exciter

Origin:		Slave

Followed by:		1 byte of data

Meaning:		At the end of BITE or when a run-time fault is detected by the PA it returns an assembly number and a fault number to the Exciter.  The Exciter breaks this up and sends it as two messages.

Data Format:		Fault number (1 byte)

Value:			5A/5B hex,   132/133 octal,   90/91 decimal

3.7  CHANNEL CATEGORY

DID:			Channel (CHANNL)

Application:		Channelized units.

Origin:		Masters and slaves.

Followed by:		Channel data byte (packed BCD).

Meaning:		From master:  A slave should change channel. From slave:  The slave is on the specified channel.  The channel DID is also used as part of the Group programming or Group reporting messages (see GROUP,UNGRUP, REPGRP DIDs).

Data format:		The channel data byte consists of two packed BCD digits.

Value:			93 hex,   223 octal,   147 decimal

DID:			Report Channel Data (REPCHA)

Application:		Channelized units.

Origin:		Master and slaves.

Followed by:		Master:  Channel data byte.

Slaves: The channel data byte, followed by a series of DIDs and their associated data bytes.

Meaning:		Controlled device should report the stored data for that channel when polled for changes.  It uses the same DID in the reply.

Data format:		The channel data byte consists of two packed BCD digits.

Value:			91 hex,   221 octal,   145 decimal

Example: The master requests channel information by

sending

..., REPCHA(51), ...

a later report from slave includes

..., REPCHA(51), FREQ3(01), ...., MODE(00)

The type and order of the DIDs in the report depend on the type of slave.

DID:			Program Channel (PROCHA)

Application:		Programmable channelized units.

Origin:		Master

Followed by:		Channel data byte (packed BCD), and a series of DIDs and their associated data bytes.  The Program Channel DID must lead the parameters to be programmed.  This is a context switching DID, which switches the context of the DIDs and data which follow it.

Meaning:		The addressed unit should change the stored data for the specified channel.  Note that some slaves may not allow programming while scanning channels.

Data format:		The channel data byte consists of two packed BCD digits.

Value:			95 hex,   225 octal,   149 decimal

Example: To program a new mode and AGC in slave's

channel 18, the master sends

PROCHA(18), MODE(00), TUNAGC(10)

All parameters following the PROCHA DID are assumed

to be programmed.  Thus, the channel programming

message is usually not embedded with other DIDs in a

long data message.

DID:			Increment Channel (INCCHA)

Application:		Channelized units.

Origin:		Masters.

Followed by:		Nothing.

Meaning:		The unit will go to the next higher sequential channel.  If not in channelized operation, or if it is already on the highest channel, it will go to the lowest channel.  If the DID is sent to a receiver which is in scan state, it will force the receiver to the next sequential channel in the scan sequence.  This is a way to force a receiver off a channel on which it has locked.  If it has certain channels locked out, it will go to the next channel which is not locked out.  If the receiver supports groups (e.g., the RF-590) and is in group scan state, it will go to the next sequential channel in the (previously) selected group.  This DID is primarily intended for use in systems of scanning receivers, with scanning under control of a master.  It can be used with broadcast addressing, to enable simultaneous scanning by multiple receivers.

Value:			DC hex,   334 octal,   220 decimal

DID:			Decrement channel (DECCHA)

Application:		Channelized units.

Origin:		Masters.

Followed by:		Nothing.

Meaning:		The slave will go to the next lowest channel.  If it is already on the lowest channel, or if it is not in channelized operation, it will go to the highest channel.  If the DID is sent to a receiver which is in scan state, it will force the receiver to the next lower channel in the scan sequence.  This is a way to force a receiver off a channel on which it has locked.  If it has channels locked out, it will go to the next lower channel which is not locked out.  If the receiver supports groups (e.g., the RF-590) and is in group scan state it will go to the next lower channel in the (previously) selected group.  This DID is primarily in-tended for use in systems of scanning receivers, with scanning under control of a master.  It can be used with broadcast addressing, to enable simultaneous scanning by multiple receivers.

Value:			DE hex,   336 octal,   222 decimal

3.8  SCAN CATEGORY

DID:			Scan Start (SCASTA)

Application:		Scanning receivers.

Origin:		Masters and slaves.

Followed by:		Channel number (packed BCD).

Meaning:		From masters:  The Scan Start DID sets the lower bound of the channels to be scanned, in channel scan state.  This DID does not actually initiate scanning, nor does it change the operating status of the receiver.  Some slaves may not allow scan bounds to be changed while they are scanning.

From slaves:  The current lower bound of scanning is reported.

Value:			E1 hex,   341 octal,   225 decimal

DID:			Scan End (SCAEND)

Application:		Scanning receivers.

Origin:		Masters and slaves.

Followed by:		Channel number (packed BCD)

Meaning:		From masters:  The Scan End DID sets the upper bound of the channels to be scanned, in channel scan state.  This DID does not actually initiate scanning, nor does it change the operating status of the receiver.  Some slaves may not allow scan bounds to be changed while they are scanning.

From slaves:  The current upper bound of scanning is reported.

Value:			E3 hex,   343 octal,   227 decimal

Example:  The master defines channels 20 through 43 to be the scan range by sending

..., SCASTA(20), ...., SCAEND(43), ...

DID:			Dwell (DWELL)

Application:		Scanning receivers.

Origin:		Masters and slaves.

Followed by:		Dwell value (one byte)

Meaning:		From masters:  The Dwell DID is used to set the dwell time for scanning.

From slaves:  The Dwell DID is used to report the currently selected dwell time.

Data format:		The dwell value byte holds two packed BCD digits indicating dwell time in tenths of seconds.  The data value FFhex is used to command or report External scan control of dwell (time per channel controlled by hardware external to the slave).

Value:			E5 hex,   345 octal,   229 decimal

Note:			Some units may not support continuously dwell and may only permit discrete values (e.g. 40H, 50H, not 43H)

Example: The master defines scan dwell time of 5.0

seconds per channel by sending

..., DWELL(50), ....

DID:			Resume Scan (RESCAN)

Application:		Scanning receivers.

Origin:		Masters and slaves.

Followed by:		Nothing, or channel number (from masters)

Nothing (from slaves).

Meaning:		From masters:  The receiver should resume scanning.  If no channel number follows, this DID will force the receiver to the next channel in the scan sequence.  If a channel number is supplied the receiver will resume scanning on that channel.

From slaves:  The receiver has resumed scanning after having been locked on a channel.  In a status message this DID is used to signify that the receiver is actively scanning (as opposed to being locked onto a signal).

Value:			E6/E7 hex,   346/347 octal,   230/231 decimal

DID:			Suspend Scan (SUSCAN)

Application:		Scanning receivers.

Origin:		Masters and slaves.

Followed by:		Masters: Nothing.

Slaves:  Channel DID and channel number of the channel on which the receiver is currently operating.

Meaning:		From masters:  The receiver should suspend scan and hold on the current channel.  It should hold there until another scan-related command (RESCAN, INCCHA, DECCHA, CHANNL, etc.) is received.

From slaves:  The receiver has suspended scan on the indicated channel, usually because it has found a signal.

Data Format:		(from slaves) The channel number consists of two packed BCD digits.

Value:			E8/E9 hex,   350/351 octal,   232/233 decimal

DID:			Lock-out (LCKOUT)

Application:		Scanning receivers with lock-out capability.

Origin:		Masters and slaves.

Followed by:		Masters: Channel number.

Slaves: Nothing.

Meaning:		From masters:  The indicated channel number should be locked out from scanning.

From slaves: The indicated channel number is locked out.  From slaves, this DID is sent only as part of a response to the Report Channel Data DID.

Data Format:		The channel number consists of two packed BCD digits.

Value:			EA / EB hex,   352/353 octal,   234/235 decimal

DID:			Unlock (UNLOCK)

Application:		Scanning receivers with lock-out capability.

Origin:		Masters and slaves.

Followed by:		From masters: Channel number.

From slaves: Nothing.

Meaning:		From masters:  The indicated channel number should be returned to the scanning list.

From slaves:  The indicated channel number is not locked out.  From slaves, this DID is sent only as part of a response to the Report Channel Data DID.

Data Format:		The channel number consists of two packed BCD digits.

Value:			EC / ED hex,   334/355 octal,   236/237 decimal

Example: The master locks channel 27 out of scan by

sending

..., LCKOUT(27), ....

If the master would request a channel report from

that slave, using the REPCHA DID, the response would

include

..., REPCHA(27), ...., LCKOUT, ....

DID:			Group (GROUP)

Application:		Channelized units with the group feature.

Origin:		Masters

Followed by:		Group data byte.

Meaning:		Indicates that the following channel DIDs are to be added (programmed) to the group specified.  Note that a group DID and data byte should therefore be followed by one or more channel DIDs with channel numbers.  The group DID serves as a context switching DID.  Slaves report group contents only by the REPGRP DID.

Data format:		The group data byte consists of two packed BCD digits.

Value:			97 hex,   227 octal,   151 decimal

DID:			Ungroup (UNGRUP)

Application:		Channelized units with the group feature.

Origin:		Masters.

Followed by:		Group data byte.

Meaning:		Indicates that the following channel DIDs are to be removed from the group specified.  An Ungroup DID and data byte may be followed by one or more channel DIDs.  Note that the Ungroup DID serves as a context switching DID.

Data format:		The group data byte consists of two packed BCD digits.

Value:			99 hex,   231 octal,   153 decimal

DID:			Report Group (REPGRP)

Application:		Channelized units with the group feature.

Origin:		Masters and slaves.

Followed by:		Group data byte.

Meaning:		From Master:  Used to request which channels are in a specified group.

From Slaves:  Used to report which channels are in a specified group.  If there are channels in the group the DID and group data byte are followed by one or more channel DIDs with their channel data bytes.  If no channels are in the group the DID is not followed by any channel DIDs.  In this application it serves as context switching DID.

Data format:		The group data byte consists of two packed BCD digits.

Value:			9B hex,   233 octal,   155 decimal

DID:			Stop Scan Threshold (SSTHRH)

Application:		Receivers and receiver controllers with remote scan capability.

Origin:		Masters and slaves

Followed by:		One byte of scan threshold data.

Meaning:		A master used this DID to send stop scan threshold data to a slave.  The slave used it to report its current stop scan threshold data value.

Data Format:		The data is encoded as 0 for minimum threshold and FF hex for maximum threshold.  All 256 data values are valid.  Note that a certain slave may use this data in a linear or logarithmic fashion.

Value:			3C/3D hex,   74/75 octal,   60/61 decimal

3.9  MODEM CATEGORY

DID:			Full/Half Duplex (FHDUPL)

Application:		RF-3466/RF-3466A

Origin:		Masters and slaves

Followed by:		One data byte.

Meaning:		From master:  Used to command modem to full or half duplex operation.

From Slave:  Used to report full or half duplex operation.

Data format:		Data byte = 00 for Half Duplex

Data byte = 01 for Full Duplex

Value:			F1 hex,   361 octal,   241 decimal

DID:			Front Panel Status (FPSTAT)

Application:		RF-3466/RF-3466A

Origin:		Masters and slaves

Followed by:		One data byte indicating the current state of the front panel LEDs.

Meaning:		From Master:  Used to request the current state of the modem's front panel LEDs.

From Slave:  Used to report the current state of front panel LEDs.

Data format:		Bit 0:	Send

Bit 1:	Signal Present A

Bit 2:	Signal Present B

Bit 3:	Data Sync

Bit 4:	Reserved

Bit 5:	Reserved

Bit 6:	Reserved

Bit 7:	Reserved

Value:			F5 hex,   365 octal,   245 decimal

DID:			Priority (PRORTY)

Application:		RF-3466/RF-3466A

Origin:		Master and Slaves

Followed by:		One byte of data.

Meaning:		Setting Priority OFF terminates the current message immediately.  KEY is an illegal DID while Priority is OFF.  Setting Priority ON will start a message if the external RTS is active.  Otherwise no message is started until a new KEY command is received after PRORTY(ON) is received.

Data format:		Data byte = 0 for Priority OFF

Data byte = 1 for Priority ON

Value:			F3 hex,   363 octal,   243 decimal

DID:			Format Configuration (FORMAT)

Application:		RF-3466/RF-3466A

Origin:		Masters and Slaves

Followed by:		One byte of data.

Meaning:		The FEC/IO board changes the operating format of the two channels as specified by the data byte following this DID.

Data format:		The valid bytes are:

Data byte		Channel A		Channel B

00				Reserved

01			RF-3466		RF-3466

02			RF-3466		FSK B

03				Reserved

04			FSK A		FSK B

05			RSM			RSM

06			RSM			FSK B

07				Reserved

RSM stands for the Robust Serial Mode option

Value:			9D hex,   235 octal,   157 decimal

DID:			0.1 Hz / 0.01 Hz (FREQD1)

Application:		RF-3466/RF-3466A

Origin:		Masters and Slaves

Followed by:		One byte of 0.1/0.01 HZ frequency data.

Meaning:		The remote controller sends this DID to set the frequency of the tone previously specified by the TONINDE DID.  The modem uses this DID to report changes in the frequency of the tone previously specified by the TONIND DID.

Data format:		The data is in the form of packed BCD where the upper nibble contains the 0.1 Hz digit and the lower nibble contains the 0.01 Hz digit.

Value:			F7 hex,   367 octal,   247 decimal

DID:			Format Select (FSELCT)

Application:		RF-3466/RF-3466A

Origin:		Masters and Slaves

Followed by:		One data byte to indicate the format being selected for all command and/or status.

Meaning:

Data format:		This is a context switching DID.  The data byte indicates which format is being selected for commands and/or status.  All DIDs and data following are interpreted based on the selected format, unless another FSELCT DID is encountered in the message.  The following data values are currently defined:

Data byte 		Format

00			Reserved

01			RF-3466

02			FSK A

03			FSK B

04			Robust Serial Mode option

Value:			9F hex,   237 octal,   159 decimal

DID:			Interleaving (INTRLV)

Application:		RF-3466/RF-3466A

Origin:		Masters and Slaves

Followed by:		One byte of data.

Meaning:		This byte is used to report or command the interleaving factor to be used.  The context of this DID is changed by the FSELCT DID.

Data format:		The current bit assignments are:

Format 01 (RF-3466)

Bit 0:	0 = short, 1 = long

Bit 1:	0 = normal, 1 = alternate

Bit 2:	Reserved

Bit 3:	Reserved

Bit 4:	Reserved

Bit 5:	Reserved

Bit 6:	Reserved

Bit 7:	Reserved

Format 02 (FSK A)

Format 03 (FSK B)

Currently no selections, command will be ignored.

Format 04 (Robust Serial Mode option)

Bit 1		Bit 2 (LSB)		Factor

0		   0			0.0 sec

0		   1			1.2 sec

1		   0			4.8 sec

1		   1			9.6 sec

Bit 2:	Reserved

Bit 3:	Reserved

Bit 4:	Reserved

Bit 5:	Reserved

Bit 6:	Reserved

Bit 7:	Reserved

Value:			8F hex,   217 octal,   143 decimal

DID:			Tone Indicator (TONIND)

Application:		RF-3466/RF-3466A

Origin:		Masters and Slaves

Followed by:		One byte of data.

Meaning:		This DID selects the tone to which the FREQ DIDs that follow apply.

Data format:		This DID and the FREQ1, FREQ0 and FREQD1 DIDs are used to report and command the following tones:

Data Byte (hex)	Tone

00			Demodulator MARK

01			Demodulator SPACE

02			Modulator MARK

03			Modulator SPACE

Value:			EF hex,   357 octal,   239 decimal

DID:			Data Rate (DATRAT)

Application:		Data communications equipment

Origin:		Master and slaves.

Followed by:		One byte of data.

Meaning:		A master uses this DID to send data rate commands to a slave.  The slave uses it to report its current data rate selection.  The equipment involved may only allow its data rate to be changed when data traffic is not taking place (i.e.  slave not 'busy').

Data Format:		The DID is followed by a data byte which selects one of the data rates supported on the device.  These selection values are specific to each equipment type supporting this DID.  Check applications information for specific equipments.

Value:			8B hex,   213 octal,   139 decimal

3.10  MISCELLANEOUS

DID:			Device Control 1 (DEVCT1)

Application:		All units

Origin:		Master and slaves.

Followed by:		One byte of data.

Meaning:		This DID is used to control one or more miscellaneous functions of a slave.  These may include on/off selection of several different device features.  A master uses this DID to send command data to a slave.  The slave uses it to report current setup.

Data Format:		The DID is followed by a data byte coded specifically for the equipment type involved.  It may be partially or completely bit-mapped.  Check applications information for specific equipments.

Value:			A5/A6 hex, 245/246 octal, 165/166 decimal

4.0  SUMMARY OF DIDS

SUMMARY OF DIDS BY VALUE

HEX		OCTAL	DECIMAL	DID

00/01		000/001	0/1		NOOP		No-operation

30/31		60/61		48/49		USBLIN	USB Line Audio Level

32/33		62/63		50/51		LSBLIN	LSB Line Audio Level

34/35		64/65		52/53		FCFLOW	FSK Center Freq Low

36/37		66/67		54/55		FCFHI	FSK Center Freq High

38/39		70/71		56/57		FSHIFT	FSK Shift & Shift Polarity

3A/3B		72/73		58/59		OPTION	Options

3C/3D		74/75		60/61		SSTHRH	Stop Scan Threshold

40		100     	64		SHIFT1	Shift 1

42		102     	66		SHIFT2 	Shift 2

44		104     	68		UNSHFT	Unshift

50/51		120/121	80/81		USBAGC	USBAGC Level

50/51		120/121	80/81		USBOUT	USB AGC Level

52/53		122/123	82/83		LSBOUT	LSB AGC Level

52/53		122/123	82/83		LSBAGC	LSB AGC Level

54/55		124/125	84/85		SIGDET	Signal Detect

58/59		130/131	88/89		PAASSY	PA Assembly

5A/5B		132/133	90/91		PAFLT	PA Fault

5C/5D		134/135	92/93		PRESTA	Preselector Status

81		201     	129		SPARE1	Spare 1

83		203     	131		SPARE2	Spare 2

85		205     	133		PWRUP	Power-up

86/87		206/207 	134/135	WHORU	Who Are You

88/89		210/211 	136/137	BWCONF	Bandwidth Configuration

8B		213  		139		DATRAT	Data Rate

8D		215     	141		TXBYPS	Transmitter bypass

8F		217		143		INTRLV	Factor Information

91		221     	145 		REPCHA	Report Channel Data

93		223		147		CHANNL	Channel

95		225     	149		PROCHA	Program Channel Data

97		227		151		GROUP	Group Program

99		231 		153		UNGRUP	Group Unprogram

9B		233     	155		REPGRP	Report Group

9D		235		157		FORMAT	Format Configuration

9F		237		159		FSELCT	Format Select Information

SUMMARY OF DIDS BY VALUE

HEX		OCTAL	DECIMAL	DID

A1		241     	161		FAULT	Fault

A3		243     	163		ERROR	Error

A5/A6		245/246     	165/166	DEVCT1	Device Control 1

A7		247     	167		LSBAUD	LSB Audio Select

A9		251     	169		RXCMST	Receiver Command/Status

AA		252		170		TXCOMD	Transmit Command

AB		253		171		TXCMST	Transmitter Command/Status

AC/AD	254/255	172/173	POWER	Power Output Level

AF		257     	175		USBAUD	USB Audio Select

B1		261     	177		BANDWT	Bandwidth

B3		263     	179		BFOHI	BFO High

B5		265     	181		BFOLOW	BFO Low

B7		267     	183		TUNAGC	Tuning method/AGC speed

B9		271     	185 		CLARFY	Clarifier

BB		273     	187		RFGAIN	RF Gain

BD		275     	189		VFOHI 	VFO High

BF		277     	191		VFOLOW	VFO Low

C1		301 		193 		FREQ3	10/1 MHz Frequency

C3 		303 		195 		FREQ2	100/10 KHz Frequency

C5 		305 		197 		FREQ1	1000/100 Hz Frequency

C7 		307 		199 		FREQ0	10/1 Hz Frequency

C9 		311 		201 		MODE	Mode

CB 		313 		203 		COMSTA	Command/Status

CD 		315 		205 		CARRIR	Carrier

CF 		317 		207 		STATE	State

D0 		320 		208 		REPCOM	Report Complete Status

D2/D3		322/323 	210/211	SELTST	Self-test (BITE)

D4 		324 		212 		KEY 		Key

D6 		326 		214		UNKEY	Unkey

D8 		330 		216 		ALTERN	Alternate Operation

DA 		332 		218 		OPNORM	Normal Operation

DC 		334 		220 		INCCHA	Increment Channel

DE	 	336 		222 		DECCHA	Decrement Channel

E1 		341 		225 		SCASTA	Scan Start Bound

SUMMARY OF DIDS BY VALUE

HEX		OCTAL	DECIMAL	DID

E3 		343 		227 		SCAEND	Scan End Bound

E5 		345 		229 		DWELL	Dwell

E6/E7		346/347 	230/231 	RESCAN	Resume Scan

E8/E9		350/351	232/233 	SUSCAN	Suspend Scan

EA/EB	352/353 	234/235 	LCKOUT	Lockout Channel

EC/ED	354/355 	236/237 	UNLOCK	Un-lock channel

EF		357		239		TONIND	Tone Indicator

F1		361		241		FHDUPL	Duplex information

F3		363		243		PRORTY	Priority Information

F5		365		245		FPSTAT	Front Panel Status

F7		367		247		FREQD1	0.1/0.01 Hz Frequency

FA/FB		372/373	250/251	TUNMOD	Tune Mode

SUMMARY OF DIDS BY NAME

DID                		HEX		OCTAL		DECIMAL

ALTERN  Alternate Operation	D8 		330			216

BANDWT  Bandwidth           	B1 		261       		177

BFOHI   BFO High    		B3 		263       		179

BFOLOW  BFO Low      	    	B5 		265       		181

BWCONF  Bandwidth Config		88/89		210/211   		136/137

CARRIR  Carrier   			CD		315       		205

CHANNL  Channel         	  	93 		223       		147

CLARFY  Clarifier        		B9 		271       		185

COMSTA  Command/Status  		CB		313      	 	203

DATRAT  Data Rate			8B 		213       		139

DECCHA  Decrement Channel	DE		336       		222

DEVCT1  Device Control 1		A5/A6 	245/246       		165/166

DWELL   Dwell			E5		345       		229

ERROR   Error 			A3		243       		163

FAULT   Fault 			A1		241       		161

FCFHI   FSK Center Freq High	36/37		66/67			54/55

FCFLOW  FSK Center Freq Low	34/35		64/65			52/53

FHDUPL  Duplex information	F1		361			241

FORMAT  Format Configuration	9D		235			157

FPSTAT  Front Panel Status		F5		365			245

FREQ0   10/1 Hz Frequency		C7		307       		199

FREQ1   1000/100 Hz Frequency	C5		305       		197

FREQ2   100/10 KHz Frequency	C3		303       		195

FREQ3   10/1 MHz Frequency	C1		301       		193

FREQD1  0.1/0.01 Hz Frequency	F7		367			247

FSHIFT  FSK Shift & Shift Polarity	38/39		70/71			56/57

FSELCT  Format Select Information	9F		237			159

GROUP   Group Program 		97		227       		151

INCCHA  Increment Channel		DC		334       		220

INTRLV  Factor Information		8F		217			143

KEY     Key           			D4		324       		212

LCKOUT  Lockout Channel		EA/EB	352/353       		234/235

LSBAGC  LSB AGC Level		52/53		122/123		82/83

LSBAUD  LSB Audio 		A7		247       		167

LSBLIN  LSB Line Audio Level	32/33		62/63			50/51

LSBOUT  LSB AGC Level		52/53		122/123		82/83

MODE    Mode  			C9		311       		201

SUMMARY OF DIDS BY NAME

DID                		HEX		OCTAL		DECIMAL

NOOP    No-operation		00/01		000/001     		0/1

OPNORM  Normal Operation 	DA		332       		218

OPTION  Options			3A/3B		72/73			58/59

PAASSY  PA Assembly		58/59		130/131		88/89

PAFLT   PA Fault			5A/5B		132/133		90/91

POWER   Power Output Level	AC/AD	254/255		172/173

PRESTA  Preselector Status		5C/5D		134/135		92/93

PROCHA  Program Channel Data	95		225       		149

PRORTY  Priority Information 	F3		363			243

PWRUP   Power-up  			85		205       		133

REPCHA  Report Channel Data	91		221       		145

REPCOM  Report Comp. Status	D0		320       		208

REPGRP  Report Group		9B		233       		155

RESCAN  Resume Scan		E6/E7		346/347   		230/231

RFGAIN  RF Gain 			BB		273       		187

RXCMST  Receiver Com./Stat.	A9		251       		169

SCAEND  Scan End Bound 		E3 		343	 		227

SCASTA  Scan Start Bound		E1		341 			225

SELTST  Self-test (BITE)		D2/D3		322/323   		210/211

SHIFT1  Shift 1 			40		100        		64

SHIFT2  Shift 2 			42		102        		66

SIGDET  Signal Detect		54/55		124/125		84/85

SPARE1  Spare 1 			81		201       		129

SPARE2  Spare 2 			83		203       		131

SSTHRH  Stop Scan Threshold	3C/3D		74/75			60/61

STATE   State   			CF		317       		207

SUSCAN  Suspend Scan 		E8/E9		350/351   		232/233

TONIND  Tone Indicator		EF		357			239

TUNAGC  Tuning meth./AGC spd.	B7		267       		183

TUNMOD  Tune Mode		FA/FB		372/373		250/251

TXBYPS  Transmitter Bypass		8D		215       		141

TXCMST  Transmitter Command/Status   AB	253			171

TXCOMD  Transmit Command	AA		252			170

UNGRUP  Group Un-program 	99		231       		153

UNLOCK  Un-lock channel 		EC/ED	354/355       		236/237

SUMMARY OF DIDS BY NAME

DID                		HEX		OCTAL		DECIMAL

UNKEY   Unkey 			D6		326       		214

UNSHFT  Unshift  			44		104        		68

USBAGC  USB AGC Level		50/51		120/121		80/81

USBAUD  USB Audio  		AF		257       		175

USBLIN  USB Line Audio Level	30/31		60/61			48/49

USBOUT  USB AGC Level		50/51		120/121		80/81

VFOHI   VFO High 			BD		275       		189

VFOLOW  VFO Low 		BF		277       		191

WHORU   Who Are You 		86/87		206/207   		134/135

5.0 GLOSSARY OF TERMINOLOGY

AFC

Automatic Frequency Control. A means for fine tuning receivers to accommodate signals varying in frequency.

AGC

Automatic Gain Control. Maintains constant output levels (of receiver audio) despite varying input signal levels.  AGC 'speeds' or time-constants refer to the reaction to the changes in signal strength.

(IF) Bandwidth

The limits to the signal's frequency range as it passes through the intermediate-frequency (IF) section of the receiver. For voice signals, this bandwidth may be several kilohertz; for CW signals, only several hundred hertz.

BFO

Beat-Frequency-Oscillator. A circuit which is primarily used to convert CW (morse code) receptions to audio notes.

BIT/BITE

Built-In-Test (-Equipment). A feature which allows a unit to perform checks on its own circuits, hopefully isolating a fault to one area.

Carrier

The signal transmitted at the selected frequency.  Since most modes have their information energy in the 'sidebands',or frequencies on either side of the carrier, the amount of signal transmitted at the carrier frequency may be selected to be suppressed relative to the sidebands.

Clarifier

A circuit which allows a radio to be shifted in receive frequency slightly above or below the selected frequency, to listen to a signal off-frequency.

Clip

Transmitter circuit which cuts the peaks off of speech audio to maintain a higher average output power, without seriously degrading the sound.

GLOSSARY OF TERMINOLOGY

Dwell

In scanning, the delay upon each channel (to listen for activity).

Group

A series of channels in any random order, selected for a specific reason (all shore stations, all ARQ users, etc.)

Key

Referring to a hand/CW/telegraph key; generally, the control signal which puts transmitter energy out over the air.

Mode

Mode of emission. Method of transmitting intelligence, such as Amplitude Modulation (AM), Frequency Modulation (FM).

Mute

Silencing of audio paths, typically outputs of a receiver.

Noise blanker

A circuit on the input of receivers that eliminates bursts of noise due to lightning, or ignition picked up by the antenna.

RF gain

Amount of amplification in the signal path of a radio.

Squelch

A system that mutes the receiver audio based on the received signal type. For example, speech squelch will keep the receiver quiet when the channel has background noise, but will unmute for received voice.

VFO

Similar to BFO with clarifier action

Vox/AntiVox

Voice operated/actuated key. The keying (activation) of the transmitter is controlled by the signal on the audio input such that it is turned on when a person speaks into the microphone. AntiVox is a measure to reject false keying due to background noise in the operator's area.

6.0 INDEX OF DIDs

0.1 Hz / 0.01 Hz (FREQD1)  60

1 KHz/100 Hz Frequency (FREQ1)  14

10 Hz/1 Hz Frequency (FREQ0)  15

10 MHz/1 MHz Frequency (FREQ3)  13

100 KHz/10 KHz Frequency (FREQ2)  14

Alternate Operation (ALTERN)  12

Audio Select (AUDIO)  41

Bandwidth (BANDWT)  25

Bandwidth Configuration (BWCONF)  27

BFO High (BFOHI)  17

BFO Low (BFOLOW)  18

Carrier (CARRIR)  39

Channel (CHANNL)  49

Clarifier (CLARFY)  29

Command/Status (COMSTA)  21

Data Rate (DATRAT)  63

Decrement channel (DECCHA)  52

Device Control 1 (DEVCT1)  64

Dwell (DWELL)  53

Error (ERROR)  7

Fault (FAULT)  10

Format Configuration (FORMAT)  60

Format Select (FSELCT)  61

Front Panel Status (FPSTAT)  59

FSK Center Freq High (FCFHI)  33

FSK Center Freq Low (FCFLOW)  32

FSK Shift & Shift Polarity (FSHIFT)  31

Full/Half Duplex (FHDUPL)  58

Group (GROUP)  56

Increment Channel (INCCHA)  51

Interleaving (INTRLV)  62

Key (KEY)  39

Lock-out (LCKOUT)  55

LSB AGC Level (LSBAGC)  30

LSB Audio Select (LSBAUD)  43

LSB Line Audio Level (LSBLIN)  23

Mode (MODE)  20

No-op (NO-OP)  4

Normal Operation (OPNORM)  12

Options (OPTION)  34, 46

PA Assembly (PAASSY)  47

PA Fault (PAFLT)  48

Power Output Level (POWER)  45

Power-up (PWRUP)  40

Preselector Status (PRESTA)  36

Priority (PRORTY)  59

Program Channel (PROCHA)  50

Receiver Command/Status (RXCMST)  24

Report Channel Data (REPCHA)  49

Report Complete Status (REPCOM)  8

Report Group (REPGRP)  57

Resume Scan (RESCAN)  54

RF Gain (RFGAIN)  25

Scan End (SCAEND)  53

Scan Start (SCASTA)  52

Self-test (SELTST)  5

Shift 1 (SHIFT1)  3

Shift 2 (SHIFT2)  3

Signal Detect (SIGDET)  38

Spare 1 (SPARE1)  11

Spare 2 (SPARE2)  11

State (STATE)  19

Stop Scan Threshold (SSTHRH)  58

Suspend Scan (SUSCAN)  54

Tone Indicator (TONIND)  63

Transmit Command (TXCOMD)  45

Transmitter Bypass (TXBYPS)  44

Transmitter Command/Status (TXCMST)  40

Tune Mode (TUNMOD)  45

Tuning Method/AGC Speed (TUNAGC)  28

Ungroup (UNGRUP)  56

Unkey (UNKEY)  39

Unlock (UNLOCK)  55

Unshift (UNSHFT)  4

USB AGC Level (USBAGC)  30

USB Audio Select (USBAUD)  41

USB Line Audio Level (USBLIN)  23

VFO High (VFOHI)  16

VFO Low (VFOLOW)  16

Who Are You (WHORU)  9